Table of Contents

# Insights Module — Technical Documentation

ARR Due Diligence · OBIWAN — Ex-ante screening for Afforestation, Reforestation and Revegetation (ARR) carbon projects.

Powered by Hansen Global Forest Change, NASA GEDI biomass, and a VM0047-aligned quantification engine.

## ⚠️ Notice to all users

This tool is a preliminary screening instrument. It is designed to be run before a complete pre-feasibility analysis, not in place of one. Its outputs identify and rank candidate areas and give order-of-magnitude carbon and credit estimates so that scarce field and modelling budgets can be pointed at the right sites. It is not a validated quantification, a Project Design Document (PDD), or a substitute for site-level measurement or assessment by a validation and verification body (VVB). Every figure it produces carries a provenance label indicating how much local measurement stands behind it; read that label before quoting any number.

## Section 1 — Theoretical Foundation & Geospatial Data Integration

The module’s central design principle is that statistics are disciplined by geography. Rather than projecting an idealised growth curve in isolation, every quantitative estimate is bounded by what two authoritative Earth-observation datasets actually record about the project’s own landscape. Two datasets do the constraining: Hansen Global Forest Change constrains where an intervention is eligible, and NASA GEDI constrains how much carbon that intervention can plausibly accumulate.

### 1.1 Hansen Global Forest Change integration — isolating eligible land

The tool uses the Hansen Global Forest Change (GFC) product (default source UMD/hansen/global_forest_change_2025_v1_13, 30 m resolution) as its single, global forest-change source. Restricting to one product is deliberate: it makes the classification reproducible and directly comparable from one country to the next, with no dependence on national land-cover layers that vary in quality and availability.

For a user-defined analysis window (T_start → T0), each 30 m pixel is assigned to one of four transition classes by a strict decision tree evaluated server-side in Google Earth Engine:

| Code | Class | Definition | Role in the screen |
| --- | --- | --- | --- |
| 11 | Stable forest | Forest in 2000, still forest at T0 | Ceiling reference (Section 1.2) |
| 22 | Stable non-forest | Non-forest throughout the window | Plantable |
| 12 | Forest loss | Forest at T_start, cleared within the window | Plantable |
| 21 | Forest gain | Non-forest that regenerated to forest | Regeneration calibration only |

“Forest” is defined by a user-set canopy-cover threshold on the year-2000 baseline (treecover2000, default 25%). Loss is dated by the lossyear band and restricted to the analysis window; a pixel is “forest at T_start” only if it was forest in 2000 and had not already been lost by T_start.

Isolation of the two plantable classes. ARR activities can only be credited on land that is genuinely non-forest at the project start. The module therefore treats only classes 22 (Stable non-forest) and 12 (Forest loss) as candidate project area:

- Stable non-forest (22) — land that has carried little or no tree cover throughout the window. This is the classic ARR opportunity: degraded pasture, abandoned cropland, open scrub available for planting or assisted regeneration.

- Forest loss (12) — land deforested within the window and not yet recovered. It is a restoration opportunity, though the cause of loss (which Hansen does not distinguish, see Section 4) should be understood before committing.

Classes 11 (Stable forest) and 21 (Forest gain) are never offered as project area in the interface. Stable forest is not ARR-eligible; it serves only as the carbon ceiling reference. Forest gain is used only to calibrate the recovery rate of the growth curve.

Hansen caveat (honest disclosure). Hansen’s gain band covers 2000–2012 only and carries no year, so class 21 cannot be restricted to an arbitrary analysis window. The module uses it exclusively to calibrate a regeneration rate, never as creditable area. Separately, Hansen “loss” means stand-replacing canopy removal, which includes plantation harvest and natural disturbance — it is a canopy-change signal, not a deforestation figure.

### 1.2 GEDI biomass integration — the localised carbon ceiling

Where Hansen answers where, NASA’s GEDI (Global Ecosystem Dynamics Investigation) answers how much. GEDI is a spaceborne LiDAR instrument that samples canopy structure and returns Above-Ground Biomass Density (AGBD) at the footprint level (Level 4A product, ~25 m footprints). Critically, GEDI is a statistical sample of the landscape, not a wall-to-wall map — and the module treats it as such.

#### Design-based estimation, not pixel arithmetic

Differencing two interpolated biomass maps confounds model error with real change and yields no defensible uncertainty. Instead, the module treats the GEDI footprints falling inside each transition class as a sample and computes a design-based estimate — a mean AGBD with a hybrid standard error that combines the sampling variance of the mean with GEDI’s own per-footprint model variance:

ȳ      = mean(yᵢ)                 estimated mean AGBD for the stratum (Mg/ha)
v_samp = s² / n                   design-based sampling variance of the mean
v_pred = mean(uᵢ²) / n            GEDI model prediction-error contribution
SE     = √(v_samp + v_pred)       hybrid standard error
95% CI = ȳ ± 1.96 · SE

Every downstream figure inherits this uncertainty, which is why the interface shows a confidence interval on essentially every biomass quantity.

#### The carbon ceiling (“carbon equilibrium”)

The key ecological construct is the carbon ceiling: the maximum above-ground biomass the project land can realistically sustain, estimated from the mean AGBD of the surrounding mature forest (the Stable-forest stratum, class 11) in the same landscape. This is the localised carrying capacity — the biomass level toward which an undisturbed stand in this climate, on these soils, at this elevation, tends to equilibrate.

The module additionally fits the recovery rate of the accumulation curve to the measured multi-year trend of locally regenerating land (the Forest-gain stratum, class 21), so both the asymptote and the speed of approach reflect this specific place rather than a textbook.

#### Why the curve must be constrained by GEDI — the ecological theory

A carbon accumulation curve is, ecologically, an approach to a stable equilibrium, not an unbounded exponential. As a stand ages, growth slows: light, water, and nutrients become limiting, self-thinning sets in, and net biomass gain asymptotes toward the carrying capacity of the site. No reforestation intervention can push a stand’s biomass durably above the level that the surrounding mature vegetation already demonstrates is sustainable, because the same climatic and edaphic limits apply.

The module enforces this directly: the resolved growth curve is clamped so that projected biomass never exceeds the GEDI-derived ceiling of the local landscape. This has three practical consequences:

- Conservatism by construction. Optimistic species-level yields are capped at what the landscape can actually hold, which is the risk-appropriate posture for a screening tool.

- Local realism over global averages. A dry-forest project is held to a dry-forest ceiling; a humid-tropical project to a humid-tropical ceiling — measured on site, not assumed.

- Transparent provenance. When GEDI cannot supply the ceiling (see below), the tool falls back to a published ecological-zone default and labels the result as indicative.

#### Provenance tiers

Because GEDI does not exist everywhere or always in sufficient density, every curve carries a provenance tier that follows the numbers all the way to the exported report:

| Tier | Meaning |
| --- | --- |
| gedi_calibrated | Both ceiling and recovery rate fit to this AOI’s own GEDI record. Strongest evidence. |
| gedi_partial | One of the two comes from GEDI, the other from published defaults. |
| ipcc_default | No usable GEDI (outside the ±51.6° orbital band, or too few footprints). Curve is literature-based — indicative only. |

The uncertainty deduction applied downstream (Section 3) escalates automatically as the tier weakens (10% → 20% → 30%), because claiming a 10% uncertainty on a curve with no site measurement would not be defensible.

## Section 2 — Reforestation Interventions & User Parameters

### 2.1 Where the defaults come from, and how to override them

The module ships two internal reference databases that seed every parameter:

- An ecological-zone table (10 FAO global ecological zones) providing, per zone, an AGB ceiling, early- and mature-phase growth rates, a root-to-shoot ratio, and a recovery-rate constant. The zone is auto-suggested from the AOI’s latitude and mean tree cover, and can be overridden.

- A species-template library (native mixed broadleaf, fast-growing pioneer, eucalyptus, teak/long-rotation hardwood, agroforestry/fruit, mangrove, natural-regeneration cohort), each carrying allometric and growth parameters.

These are screening approximations informed by IPCC 2019 Refinement (Vol. 4, Ch. 4), not verified Tier-2 values. They exist to give a sensible curve before the user supplies better numbers.

User-supplied data. Every field is exposed in the interface. A user with species-specific or measured parameters can:

- Select a template as a starting point, then edit any field (max AGB, growth-rate constant, stem density, mortality, wood density, root:shoot, harvest cycle).

- Build a multi-cohort mix (up to five species), each with its own parameters and an area fraction; fractions are normalised to sum to 1.

- Enable GEDI calibration, which overrides the template ceiling (and, where possible, the recovery rate) with values measured on the AOI itself.

Where local GEDI calibration and user parameters disagree, the resolved ceiling is taken from the strongest available evidence and each species’ own maximum AGB is scaled to that ceiling, so a locally-measured carrying capacity is never silently ignored.

### 2.2 Parameter reference

For each parameter: what it is, why it matters, and how it operates in the data flow (its effect on the curve’s shape or asymptote).

#### Intervention Type

What it is. The land-management strategy. The module implements three intervention archetypes, each mapped to a growth-model family and a set of establishment assumptions:

| Value | Growth model | Establishment | Notes |
| --- | --- | --- | --- |
| Active planting | Logistic | Bare ground, ~1 yr lag, defined stem density | Fastest early accrual, highest establishment cost and mortality risk |
| Assisted Natural Regeneration (ANR) | Recovery curve | Existing rootstock/seed bank, head-start (“advancement years”), no planting | Slower canopy closure but non-zero year-0 stock |
| Mixed / enrichment | Blend | ANR matrix with enrichment planting in gaps | Weighted by an ANR area fraction |

Agroforestry is supported, but as a species/composition choice (the agroforestry template: low stem density, low ceiling, livelihood co-benefits) rather than a fourth intervention toggle. Model it by selecting active planting (or mixed) with the agroforestry cohort at an appropriate density.

Why it matters. The intervention determines the fundamental shape of the curve. Active planting starts from zero and follows a sigmoid establishment curve. ANR starts from standing biomass and follows a saturating recovery toward the ceiling — crediting only the year-over-year increment, never the pre-existing stock (an early modelling error that this tool explicitly corrects). This distinction is material to both the timing and the additionality of credits.

How it operates in the data flow. The intervention preset supplies (a) the growth-model family, (b) an establishment lag that shifts the curve right, (c) an advancement-years head-start for ANR, and (d) multiplicative modifiers on both the growth rate and the ceiling. It is the first lever applied when the curve is resolved.

#### Species Composition

What it is. The mix of species cohorts on the project area — from a monoculture (a single cohort at 100% area fraction) to a mixed native planting (several cohorts, each with an area share). Each cohort carries its own growth model, maximum AGB, growth-rate constant, stem density, mortality, and wood density.

Why it matters. Composition affects two things the carbon accounting cares about:

- Early-stage growth rate. Fast-growing pioneers close canopy and accrue biomass quickly but plateau at a lower ceiling; long-rotation hardwoods start slowly but reach a higher, more stable carbon density.

- Long-term carbon-density stability. Mixed native stands are generally more resilient (pest, disease, drought, fire) and hold a more stable long-term carbon density than a monoculture, which is exposed to correlated failure and, if harvested, to periodic stock resets.

How it operates in the data flow. Each cohort’s carbon curve is computed independently and then area-weighted by its fraction to produce the stand-level curve. A cohort’s own maximum AGB is treated as its share of the site’s potential and rescaled to the resolved (GEDI-constrained) ceiling, keeping the mix internally consistent with the measured carrying capacity.

#### Initial Planting Density (trees/ha)

What it is. The number of stems established per hectare (typical range ~400–2,500 trees/ha; 0 for pure natural-regeneration cohorts). Agroforestry sits low (~400); native restoration around 1,100; dense pioneer or mangrove plantings higher.

Why it matters. Density governs the slope of the early-stage accumulation curve — how quickly the site captures growing space and closes canopy — and interacts with mortality to determine effective stocking. Too low and the site under-utilises its growing space for years; too high and self-thinning and competition erode the benefit.

How it operates in the data flow. Density scales the establishment phase of planted (logistic) cohorts and is the base against which the survival/mortality discount is applied. Because the stand-level ceiling is fixed by the site’s carrying capacity, density chiefly affects how fast the curve climbs, not its asymptote — a denser planting reaches a given carbon level sooner but converges to the same ecological ceiling.

#### Expected Survival Rate (%)

What it is. The proportion of established stems that persist. In the module this is expressed through two mortality parameters rather than a single number: a first-year (establishment) mortality and an annual mortality thereafter. Effective survival is (1 − establishment mortality) × (1 − annual mortality)^years, applied up to canopy closure.

Why it matters. Survival acts as a discount factor on the early biomass-accumulation slope. Establishment losses are the leading cause of ARR under-performance; a curve that assumes perfect survival systematically over-credits the first decade. Disturbance settings (fire risk, grazing pressure) feed directly into mortality, because browsing and fire are the dominant real-world causes of seedling loss.

How it operates in the data flow. Survival reduces effective stocking during the establishment phase, damping the early slope. It is applied only up to canopy closure, by design: a stand’s maximum AGB is a stand-level asymptote that already embeds self-thinning, so compounding per-stem mortality across the full rotation would double-count losses and drive the curve unphysically downward in later decades. Mortality therefore discounts early accrual, not the mature ceiling.

### 2.3 Modifiers: site, climate, and disturbance

On top of intervention, composition, density and survival, four site-condition levers apply multiplicative modifiers to the growth rate and the ceiling (and additive modifiers to mortality):

- Site index (0.5–1.5) — productivity relative to the zone average.

- Soil quality (degraded / moderate / good).

- Water stress (none / seasonal / severe).

- Fire risk and grazing pressure (none/low → high) — primarily raise mortality.

These compound (a degraded, drought-stressed site is worse than either alone) and are clamped to sane bounds so that a stack of pessimistic settings cannot drive the curve to zero. All of this remains subordinate to the GEDI ceiling: modifiers move the curve within the envelope the landscape allows, never above it.

## Section 3 — Decision Logic & Data Flow

### 3.1 The spatial screening logic

Before any carbon is modelled, the tool decides which land is eligible. The AOI (a KML boundary, ≤ 1 MB) is intersected with the Hansen product, classified into the four transition classes, and filtered so that only the two plantable classes can become project area. Per-class areas are computed with a latitude-corrected pixel-area sum (never a naive pixel count), so hectare figures are correct at any latitude.

flowchart TD
    A[User AOI boundary<br/>KML ≤ 1 MB] --> B[Clip Hansen GFC<br/>to AOI]
    B --> C{treecover2000<br/>≥ canopy threshold?}
    C -- Yes --> D{Lost within<br/>T_start → T0?}
    C -- No --> E{Regenerated<br/>gain flag?}
    D -- Yes --> F["12 · Forest loss<br/>(PLANTABLE)"]
    D -- No --> G[11 · Stable forest<br/>ceiling reference]
    E -- Yes --> H[21 · Forest gain<br/>recovery calibration]
    E -- No --> I["22 · Stable non-forest<br/>(PLANTABLE)"]

    F --> J[Per-class area<br/>pixelArea sum, ha]
    G --> J
    H --> J
    I --> J

    J --> K{Class is<br/>22 or 12?}
    K -- Yes --> L[Eligible project area<br/>selectable in UI]
    K -- No --> M[Reference / calibration only<br/>never creditable]

    classDef plant fill:#0f7b47,stroke:#0b5c36,color:#fff;
    classDef ref fill:#374a41,stroke:#26312b,color:#e7eee9;
    class F,I,L plant;
    class G,H,M ref;

### 3.2 The carbon curve calculation engine

Once eligible area is known, the engine resolves a single stand-level carbon curve from five independent levers, constrains it to the GEDI ceiling, ages the project cohorts over the crediting period, and applies the VM0047-aligned deduction chain to reach a net emission- reduction (ER) estimate with a sensitivity band.

flowchart TD
    subgraph INPUTS[User & data inputs]
        I1[Intervention type]
        I2[Species mix + density]
        I3[Survival / mortality]
        I4[Ecological zone + site modifiers]
        I5[(GEDI footprints<br/>design-based AGBD)]
    end

    I5 --> CAL{GEDI usable?<br/>in coverage & enough footprints}
    CAL -- Yes --> C1[Ceiling = mean AGBD of<br/>local mature forest, class 11]
    CAL -- Yes --> C2[Recovery rate fit to<br/>regenerating land, class 21]
    CAL -- No --> C3[Ceiling & rate from<br/>IPCC zone defaults<br/>tier = indicative]

    I4 --> C1
    C1 --> R[Resolve ceiling & rate]
    C2 --> R
    C3 --> R

    I1 --> S[Per-cohort growth model<br/>logistic / recovery curve]
    I2 --> S
    I3 --> S
    R --> S

    S --> W[Area-weight cohorts<br/>→ stand curve]
    W --> CLAMP[[Clamp: biomass ≤ GEDI ceiling]]
    CLAMP --> AGE[Age cohorts over<br/>crediting period]

    AGE --> D1[− Baseline removals<br/>from project stratum trend, ≥ 0]
    D1 --> D2[− Leakage · Performance benchmark]
    D2 --> D3[− Uncertainty<br/>escalated by provenance tier]
    D3 --> D4[− Non-permanence buffer]
    D4 --> OUT[Net ERs / VCUs<br/>+ Base / Conservative / Optimistic band]

    classDef gedi fill:#0f7b47,stroke:#0b5c36,color:#fff;
    classDef ipcc fill:#8a4b0c,stroke:#5c3208,color:#fff;
    classDef out fill:#3d6fb0,stroke:#2b4f80,color:#fff;
    class C1,C2 gedi;
    class C3 ipcc;
    class OUT out;

Reading the engine. The two branches out of “GEDI usable?” are the provenance fork: the green path (site-calibrated) and the amber path (IPCC-default, indicative). The double-bordered clamp node is where the ecological-ceiling constraint from Section 1.2 is enforced. The deduction chain at the bottom is where screening-level additionality and permanence are represented conservatively (Section 4 and FAQ Q4).

## Section 4 — Use Cases & System Limitations

### 4.1 Primary use case

A rapid, first-pass screening and pre-feasibility evaluation tool. Its job is to help a project developer, investor, or analyst answer, in minutes and at zero field cost:

- Is there enough eligible (Stable non-forest / Forest loss) land in this AOI to be worth pursuing?

- What is the order-of-magnitude carbon and credit potential, with an honest uncertainty band?

- How do intervention choices (planting vs. ANR, species, density) change that potential?

- Which of several candidate areas should receive the limited budget for full field measurement and methodology-compliant modelling?

It is a funnel and a triage instrument — a way to spend expensive due-diligence effort only where the cheap screen says it is warranted. It sits before a full pre-feasibility or site-level feasibility study, not in place of it.

### 4.2 Key limitations

Reliance on global default datasets. Hansen and GEDI are global products with genuine, well-documented localisation discrepancies:

- Hansen may mis-time or mis-classify change in cloud-prone, dry, or plantation landscapes, and its “loss” does not distinguish deforestation from harvest or natural disturbance.

- GEDI is a sample, is unavailable above ~51.6° latitude, and can be sparse over small AOIs — in which cases the tool falls back to zone defaults and labels results as indicative.

- Both are subject to their own measurement and model error, which the tool propagates but cannot eliminate.

Simplified, screening-level carbon model. The engine is a transparent, screening-grade model — not a full Tier 3, methodology-specific accounting. Specifically:

- The baseline is approximated (from the project stratum’s own historical trend, clamped to be non-additional), not a formally modelled Verra/Gold Standard counterfactual.

- Additionality, leakage, and non-permanence are represented as conservative deductions, not as the evidence-based assessments a methodology requires (see FAQ Q4).

- Carbon pools are simplified (above- and below-ground woody biomass by default; SOC, deadwood, and litter optional and unverified).

- The GEDI standard error slightly understates total uncertainty because it approximates, rather than fully propagates, GEDI’s model covariance term. This is one reason the tool applies its own sample-size-aware uncertainty floor.

In short: the outputs are defensible as a screen and indefensible as a submission. Treat the sensitivity range, not the point estimate, as the result, and never use these figures for credit issuance, sales, or investment commitment without the subsequent feasibility work they are meant to prioritise.

## Section 5 — QA / FAQ

Q1 — Is there another version of this tool available?

Yes. This release is an early, evolving version, deliberately not framed as a finished product. It emerged from an iterative development process and is expected to grow — more datasets, richer intervention modelling, and tighter methodology alignment are on the roadmap. We see tooling in this space as a collaboration: user feedback from real screening work directly shapes what gets built next. If you have parameters, local data, or workflow needs the tool does not yet cover, that input is welcome and actively wanted.

Q2 — Why do the carbon estimations from this tool look different from my own field data or models?

This tool uses a carbon-equilibrium approach. Its projected carbon stock will never exceed the ecological carrying capacity defined by the local GEDI biomass data (the mean biomass of the surrounding mature forest). Your ground-measured results or local models may show higher potential yields under optimal management — and they may well be right for a specific, well-tended stand. This tool deliberately implements a conservative, ecologically constrained ceiling for risk management: at the screening stage it is safer to under-promise. Remember that this tool precedes a full pre-feasibility or comprehensive site-level feasibility study, where localised parameters and primary field data are fully integrated and where a higher, well-substantiated yield can legitimately be modelled.

Q3 — What is the spatial resolution and update frequency of the underlying data?

- Hansen Global Forest Change — 30 m pixels, updated annually. The default source is the 2025 v1.13 collection, whose loss record extends through 2024; older collections remain selectable for reproducibility.

- GEDI L4A biomass — ~25 m footprints, collected from 2019 onward as an orbital sample (not wall-to-wall), between roughly 51.6° N and 51.6° S. The tool aggregates the available multi-year footprint record per transition class.

Because these cadences differ and GEDI is a sample, small or recently-changed AOIs can have thin footprint counts. The provenance tier on every output tells you when this has forced a fallback to defaults. Re-running after a new Hansen release, or over a larger AOI, can improve both the classification currency and the GEDI sample size.

Q4 — How are additionality and leakage handled at this screening stage?

Conservatively and approximately — not as a methodology-compliant assessment. At the screening stage:

- Additionality is approached two ways. Observed historical additionality (whether the land is already gaining biomass) is reported as a sanity check. For the ex-ante estimate, the baseline is taken from the project stratum’s own historical trend and clamped to be non-additional (≥ 0), so any pre-existing biomass gain is subtracted from the project’s gross removals every year rather than credited. This is a deliberately cautious proxy for a formally modelled counterfactual, not a substitute for one.

- Leakage is applied as a user-set percentage deduction over a configurable period, a placeholder for the activity-shifting and market-leakage analysis a full methodology requires.

- Non-permanence is represented by a buffer-pool deduction, and uncertainty by a deduction that escalates automatically when the carbon curve is not backed by local GEDI measurement.

A rigorous additionality demonstration (regulatory surplus, barrier analysis, common- practice) and a quantified leakage assessment are part of the feasibility stage that this screen is designed to precede, not replace.

Q5 — Can I use these outputs for credit issuance, sales, or an investment decision?

No. These are ex-ante screening figures. They are appropriate for prioritising sites, sizing an opportunity, and deciding where to spend field budget. They are not a validated quantification and must not be used for issuance, forward sales, or a final investment commitment without the subsequent methodology-compliant feasibility work.

Q6 — My project area is outside GEDI coverage (high latitude). Does the tool still work?

Yes, with a clearly-labelled caveat. Outside the ±51.6° GEDI band, the carbon curve cannot be calibrated from local LiDAR and falls back to IPCC ecological-zone defaults; results are tagged ipcc_default (indicative only) and carry a larger uncertainty deduction. The spatial screen (Hansen classification, eligible-area accounting) works everywhere on Earth regardless of GEDI coverage.

Q7 — What does the “provenance tier” on my results mean, and why should I care?

It states how much local measurement stands behind the carbon numbers: gedi_calibrated (site-measured ceiling and rate), gedi_partial (one of the two), or ipcc_default (literature only). It is the single most important label on any output: a gedi_calibrated estimate is a defensible screen, while an ipcc_default estimate is an order-of-magnitude indication. Always quote the tier alongside the number.

### Document control

| Field | Value |
| --- | --- |
| Module | /insights — ARR Due Diligence · OBIWAN |
| Status | Screening tool (pre-feasibility). Not for issuance or investment commitment. |
| Data sources | Hansen GFC (UMD) · GEDI L4A (NASA/UMD) |
| Methodology alignment | VM0047-aligned, screening-level (not a validated quantification) |
| Default parameter provenance | IPCC 2019 Refinement, Vol. 4, Ch. 4 — verify before any submission |